If MK902II has following issues, please solve these issues by recovery/update method.

- If you need update to the latest firmware,

- After boot, there is no signal;

- There is only boot screen;

- System running very slow, crashing;

- Can’t find the wireless network equipment, or can’t connect wireless;

- When installation apps, player loss power unexpected and cant boot up.

- TF card update failed and can’t boot again etc.

Recovery/update steps:

First, download recovery tool,driver and recovery firmware(img file) from official website.

Download, decompression the folder and find to double click, below screen will pop out, then add img file.

Second, connect the mini pc with computer.

- Prepare an USB cable (male USB to male USB cable); insert one USB port to computer usb host.

- Prepare a thin metal stick; press the recover button by the stick according below method.

- Don’t loose the stick, at same time, insert the another USB port to the unit USB  slave  port, see below picture. After 3-5s, loose the stick.

Third, install the USB driver.

Press“computer”with right key, computer---management--device management--other device, pls check below picture.

Double click unknown device.

Press update driver,Click browse for driver software on your computer. And choose the correct driver.

If your computer is WIN7 32bit, choose x86--win7; if it is WIN7 64bit, choose x64--win7.

Press next, will pop up below window, Click on the image below red circle option.Click install.

Then pls wait for installation.See below picture.

Now the driver was installed successfully. Press close.

After successful installation, the batch tool will connect the MK902II automatically.If it cant connect automatically, pls reconnect the MK902II to the computer by usb cable again. See step 1. No need to do step2.

Then Click “restore”for recovery, as below .

This procedure will cost 3minutes also, pls wait patiently, during recovery/upgrade, don’t remove the USB cable.

When this prompt window appears, then means recovery is successful, pull out USB cable, then total process is finished.

Notice:

1.Don’t power off during updating or recovering.

2.First turn on the unit after recovery/update will cost more long time, and even occur some lag issue, it’s normal since internal apk file need be decompressed and occupy RAM, after 5 minutes, then system running well. So don’t worry about that.